华为E3372 USB-4G用在Nano小蓝盒pi-star实测可用

2018/1/25

首次分享在QQ群：320533571 博客 www.aprspi.org

RiKu Bister instructions how setup the 4g modem, this is cdc_ether, usb-ethernet compatible modem, if you have ppp modem you need use wvdial(or something similar) and setup automated connection scripts for that, and that is different thing, in case of that it is modem depend how setup goes.
also might search google if your modem has support of ether mode. its easiest way use ethernet usb compatible mode. But its linux, everything is possible :D :D

first you want to add this repository:
sudo nano /etc/apt/sources.list.d/backports.list
there insert line: deb http://ftp.de.debian.org/debian jessie-backports main

sudo apt-get update
sudo apt-get install libusb-1.0-0 #youprobably have this already installed.
sudo apt-get -t jessie-backports install usb-modeswitch usb-modeswitch-data

plug modem and after this it should work, reboot if need,
make sure you have lsmod loaded modules cdc_ether and usbnet
also check dmesg if that will detect how it soppose to.
make sure you have writable filesystem, if not type this command:
mount / -o remount,rw